Quick-Start Guide to XFree86 Setup

Joe Moss

13 November 1995

Current releases of XFree86 include several tools that can help to

automate the process of server configuration. Much of the existing

documentation, however, describes how to do the job manually, includ-

ing many technical details.  For those users with esoteric hardware or

with the desire to get their hands dirty under the hood, this is

great, but many users are using common hardware and just want to get X

up and running quickly.  This guide is for them.

1.  Before You Start

There are a few bits of information that you will need to have before

you can setup the server:

The model name of your video card

Make sure you know the exact model name of the card.  It may

help to also know the graphics chipset, RAMDAC, and clock chip

used on your card.

The amount of memory on your video card

Find out how many megabytes of RAM are on your video card.

Your monitor's specifications

Specifically, you need to know the horizontal sync rate(s), and

vertical refresh rate(s).  These are important!  Consult your

monitor's manual.

The protocol your mouse uses

Different mice use different methods to communicate their

movements to the computer.  Some mice may use more than one (for

example, they may have a switch on them to select between two

options).  If you can't find this out, you can try assuming it

uses a common protocol, such as that used by Microsoft mice.

2.  What to Do - An Overview

Okay, now that you have the necessary information (you do have it

don't you? the previous section is a necessary step), you basically

have three steps you need to do:

o  Run xf86config and answer the questions

o  Fix up the generated XF86Config

Why did I list only two steps, after I said there were three?  Well,

that's because after doing the first two (and step two might not even

be needed, if you're lucky) you should be able to run the X server, it

just might not display as well as it could.  If by chance, you can't

get the server working after performing the above steps, see the trou-

bleshooting section.

To get things looking just right, you'll want to do step three:

o  Use xvidtune to improve the display

All of these steps are explained in detail in the following sections.

If you're in a real hurry you can probably skip the section explaining

xf86config, because it's pretty good about explaining itself when you

run it.

3.  Running xf86config

From a text screen, run the xf86config program.  This program should

be run as root (although not absolutely necessary, it will allow

xf86config to do more of the work for you).  You can press your

interrupt character (usually Control-C or perhaps Delete), at any time

to stop the program, if you need to.  You can just start it over

again.

The xf86config program provides instructions on screen as to what you

need to do.  Following are some notes that document the various stages

in the process. They should help you get through the process quickly

and provide some documentation for those people who like to know what

they're getting themselves into, before running a program.

3.1.  The intro screen

First, xf86config begins by telling you a few things like the fact

that it can help you setup an XF86Config file or that you can do the

job yourself with an editor.  Just read what it says and press Enter

when done.

3.2.  Getting your PATH right

The program will next check that you have the directory /usr/X11R6

(the standard installation directory) on your system and tell you that

it needs to be in your PATH environment variable.

It will also check if you have the /usr/X386 directory as used by

older (pre 3.0) versions of XFree86.  If by chance you do, it will

warn you that /usr/X11R6 must be before /usr/X386 in your PATH.

If everything is okay, just press Enter and go on, otherwise press

Control-C to exit and make any necessary changes and restart

xf86config.

3.3.  Mouse setup

Pick the mouse type from the menu and enter the name of the device to

which mouse is connected, as directed.

If you are using an OS (e.g. SVR4, SCO) that has a built in mouse

driver that the Xserver could use, you'll need to edit the XF86Config

file to setup your mouse, so just pick any mouse from the list and

press enter when asked for the device.

If you don't know which protocol your mouse uses, you'll just have to

guess (the xf86config program will give you some hints as to which

might be most likely) and then see the troubleshooting section if it

doesn't work when you run the server.

3.4.  Keyboard setup

Simply answer yes to the question regarding keyboard setup.

If there is some reason you need to use the right-alt and control keys

for something else, you can enter no.

3.5.  Monitor setup

Setting up a monitor consists of entering the specifications of your

monitor and a description of the model and manufacturer.

You are first asked for the horizontal sync rate.  It is VERY

important to enter the correct value(s) from the manual. If one of the

ranges given matches the rate of your monitor, then pick it, otherwise

pick custom and enter the values from your manual.

Next is the vertical refresh rate. Again, it is VERY important that

this parameter be specified correctly.  Enter it in a manner similar

to the horizontal sync rate.

If either rate is mis-specified, it can result in damage to your

monitor.

Finally, you are asked for an "identifier", your monitor manufacturer,

and model.  You can just press enter to get through these quickly.

3.6.  Selecting your card

You are next asked if you would like to view the database of cards.

Picking your card from the list will cause the answers to the

questions in the next two sections to be filled in for you and so can

save a little time.

If your card does not appear in the list, just press q and enter to

skip on to the next step - where you'll have to answer the questions

yourself.

3.7.  Server selection

If you selected your card in the previous step, then server selection

is easy - just use the recommendation from the database.

If you have a card which uses one of the chipsets for which a specific

server exists (Mach8, Mach32, Mach64, AGX/XGA, 8514/A, S3, ET4000/W32,

I128, P9000) you'll want to pick the accel option.

Otherwise you'll probably want to use the SVGA server.

Next, answer yes when the program asks if you want it to set the

symbolic link for you.  If you picked the accel option, you'll also

need to indicate which particular accelerated server to link to.

3.8.  Screen/Video configuration

Pick the appropriate option from the list to indicate the amount of

memory on your video card.

Then you are asked to provide and identifier, the manufacturer, and

the model of your card.  You can just press enter to skip through

these, if you wish.

Next, the program will ask for the type of RAMDAC and Clockchip on

your card.  If your card was in the database, you should just to tell

it to use the values from the database.

If you don't have one of the listed RAMDACs or Clockchips on your

card, just press enter when asked what type you have.  If you do not

have a programmable clock chip, the program will next attempt to probe

to find out what clock rates are supported by your clock chip.

3.9.  Mode Selection

Now you get to tell the program which video modes you would like to be

able to run.

The program will show you the common modes that should work with your

card (some might not work with your monitor, but if you've correctly

specified the monitor's sync rates, the X server will just ignore them

when it runs).

You could just accept the settings as they are given, but you'll

probably wish to reverse the order.  For example, if you have a card

with 1 Meg RAM, it will list the modes

"640x480" "800x600" "1024x768" for 8bpp

Select 1 to change the settings for 8bpp and the type 432 to select

the reverse order.

When you've select the modes, in the order you wish, select option 4

to continue.

3.10.  Creating the XF86Config file

The program will now ask if you would like to write the configuration

settings you've selected to the file XF86Config. Answer yes.

3.11.  Some final notes

Lastly, the program tells you that it's finished its part of this

process and counsels you to check the file before using it.  The next

section covers the changes that are most likely to be needed.

4.  Fixing the XF86Config file

Use an editor to look at the XF86Config file. Here are some things

that may need to be changed:

o  If you are running an operating system which has built-in mouse

support, you'll want to change the Pointer section.

Specifically, you should set the Protocol to OSMouse (SCO) or

Xqueue (SVR4, some SVR3) and you should remove the Device line.

o  If you are running a system with the Xqueue event driver and

would like to use it, change the Protocol setting in the

Keyboard section to Xqueue.

Once you are satisfied that the configuration is correct, copy the

XF86Config file to /usr/X11R6/lib/X11 and run the 'startx' command.

You should now have a running X server.  If it's running but the

display doesn't look as good as you think it should (i.e.  it doesn't

fill the whole screen, it's off-center, it's wrapping around on one

side, etc.) see the section on xvidtune.  If there is some other

problem, see the troubleshooting section.

5.  Running xvidtune

If you need to make adjustments to the video display, xvidtune is the

tool to use.

Simply enter the command xvidtune from a shell prompt within an xterm.

Read the warning and click on OK.  Next click on the Auto button.

Now click on whatever combination of Up/Down/Left/Right

Shorter/Taller/Wider/Narrower is need to adjust the display to your

liking.

If you are using a recent S3-based card there will be some extra

buttons and entries at the bottom (InvertVCLK, EarlySC, and Blank

Delays). These can help solve problems of the display wrapping around

a few pixels.

Once the display has been adjusted properly, press the show button to

printout the correct ModeLine to put in the XF86Config to make the

server always use the current display settings.  To aid in copying

this information to your XF86Config file, the modeline is also made

the current selection allowing you to just paste it into your editor.

If you would like to adjust your other modes, you can click on the

Next and Prev buttons to switch modes.

When you are through using xvidtune simply press on the Quit button.

6.  Troubleshooting

Since you're reading this, something must not have gone the way you

had hoped (or else you just really enjoy reading).

Below are listed some common problems that may occur during

configuration and some hints for solving them.  However, there are

just too many different combinations of hardware and software

configurations, and, well, just too many things that can go wrong, for

this document and the tools it documents, to cover every case.

If after trying the steps in the previous sections and checking the

hints in this section, you still are unable to get your system

working, you'll have to read the full documentation.  Read the README

file for your card and OS (if they exist), the XFree86 Configuration

Guide (README.Config), and the XF86Config man page.

You should also look at the XFree86 FAQ <http://www.XFree86.org/FAQ>

for more up-to-date information, especially if you are trying to

configure a fairly new card.

If all else fails, you can try posting a message to

comp.windows.x.i386unix or comp.os.linux.x or send email to

XFree86@XFree86.org.

6.1.  The mouse doesn't move correctly, it stays in one area of the

screen

You've selected the wrong protocol for your mouse. Try a different

one.

6.2.  The server doesn't start, it says the mouse is busy.

Well, it's probably right.  This most often happens on Linux systems

that have gpm running.  Kill the gpm process and try startx again.

6.3.  The middle button doesn't work.

There's no easy answer to this one.  It's a lot of trial and error.

You need to make sure you're running the right protocol for your

mouse.

Many three button mice are "dual protocol" which means that they have

both a 2-button and 3-button mode. The way to get the mouse to switch

into 3-button mode (which usually then uses MouseSystems protocol)

varies between different models.

You may need to slide a switch on the mouse or hold down the middle

button when starting the server.  Other methods of switching modes can

be done by the server, you just have to find the right combination of

settings for your mouse.  See the Pointer section of the XF86Config

man page for a complete list of settings.

6.4.  The display is shifted to the left/right/top/bottom

See the section on xvidtune.

6.5.  I don't appear to have xf86config or xvidtune on my system

Hmmm. Three possibilities:

1. You have a version of XFree86 that is older than 3.1.2.  If this

is the case then you probably aren't reading this document

either, because it wasn't included in 3.1.2 and earlier

releases.  Please upgrade to the latest version.

2. Your PATH is not set correctly. Make sure it includes the bin

directory for the XFree86 binaries (usually, /usr/X11R6/bin

3. You don't have a complete installation of XFree86.  Go back to

wherever you got XFree86 and get the missing pieces.

Generated from XFree86: xc/programs/Xserver/hw/xfree86/doc/sgml/QStart.sgml,v 3.1 1995/12/28 03:49:32 dawes Exp $

$XFree86: xc/programs/Xserver/hw/xfree86/doc/QuickStart.doc,v 3.1 1995/12/28 03:52:16 dawes Exp $